## HYPSTAT: Hydrogen Example Study Documentation

The case study examines the infrastructure network for hydrogen demand, production, storage, and transmission across various nodes, each with distinct characteristics and demand profiles. The demand nodes evaluated include a connection point, industrial site, city, inland area, suburb, and coastal area. Each node’s potential to support renewable energy sources—such as onshore wind, offshore wind, solar, and nuclear energy—is assessed, along with their grid connectivity, export/import capacity, and storage capabilities. The case study is designed to demonstrate the modeling capabilities of the Hydrogen Production, Storage, and Transmission Analysis Tool (HYPSTAT).

This documentation provides an overview of key concepts, formulas, and methodologies needed to derive essential results from HYPSTAT for hydrogen system balancing and dynamics. By analyzing direct model outputs, such as renewable, electrolyzer, transmission, and storage builds and locations, as well as by calculating Total CAPEX (Capital Expenditure), OPEX (Operational Expenditure), Levelized Hydrogen Cost (LCOH), Levelized Cost of Electricity (LCOE), and Capacity Factors for both electrolyzers and renewable resources, users can evaluate the technical and economic performance of hydrogen production facilities based on renewable energy sources and optimize the system for cost-effective, large-scale hydrogen production.

Case Design:

| Node | Descriptor | Demand | Onshore Wind | Solar | Offshore Wind | Nuclear | Grid | Export | Import | Geologic Storage | Tank Storage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Connection point | NO | YES (Good) | YES (Poor) | NO | NO | YES (Wholesale, lower) | YES | YES | NO | YES |
| B | Industrial site | YES | YES (Good) | YES (Poor) | NO | YES | YES (Fixed industrial) | NO | NO | NO | YES |
| C | City | YES | NO | NO | NO | NO | NO | NO | YES(?) | NO | NO |
| D | Inland | YES | YES (Poor) | YES (Good) | NO | NO | YES (Wholesale, higher) | NO | NO | YES (20 kT) | YES |
| E | Suburb, peaker plant | YES | NO | YES (Good) | NO | NO | YES (Fixed residential) | NO | NO | NO | YES |
| F | Coastal | YES | NO | YES (Good) | YES | NO | YES (Wholesale, higher) | NO | NO | NO | YES |

Node A (Connection Point): This node serves as a major junction in the hydrogen network, with no local demand but significant import and export capabilities. It has good onshore wind resources and modest solar potential. It connects to the wholesale grid at a lower price and has tank storage facilities.

Node B (Industrial Site): Characterized by a relatively large and flat demand profile, this site supports both onshore wind and solar (though with poor offshore wind potential). It pays a fixed industrial price, allowing it to manage stable hydrogen production and storage.

Node C (City): The city experiences medium demand, peaking in winter for heating and maintaining a consistent baseline for fleet vehicles. It lacks renewable energy resources, grid electricity, or hydrogen storage facilities to meet local H2 demand; all H2 is expected to be met by imports from other regions.

Node D (Inland): With small demand primarily for heavy-duty fuel cell electric vehicles (HD FCEVs), this node has poor onshore wind resources but good solar potential. It connects to the wholesale grid at a higher price and has significant geologic storage (20 kT) alongside tank storage.

Node E (Suburb, Peaker Plant): This node has a large demand, especially in summer for power generation. It has good offshore wind potential and a fixed residential grid connection, though it lacks geological storage.

Node F (Coastal): The coastal node features medium demand for port operations with relatively flat consumption patterns. It harnesses good offshore wind resources and is connected to the wholesale grid at a higher price. Tank storage is available, but there is no geologic storage.

The system network and the relative locations of each node are shown above. This structured network serves as input for potential locations and connections. The HYPSTAT evaluation aims to identify optimal configurations and strategies for hydrogen production, storage, and distribution, ensuring the lowest system-wide costs (objective function).

Technologies and Cost Reference:

| Type | Tech | Cost Reference |
| --- | --- | --- |
| Renewable | Onshore wind | NREL ATB moderate |
| Renewable | Offshore Wind | NREL ATB moderate |
| Renewable | Solar | NREL ATB moderate |
| Renewable | Nuclear | LCOE from H2@Scale |
| Renewable | Grid | NREL Cambium (Mid-case) for US national in hourly resolution (column name: ‘total_cost_enduse’) |
| H2 production | PEM Electrolyzer | H2A |
| Storage | Geologic | US number similar to NYSERDA, reference check with Jesse |
| Storage | Tank | US number similar to NYSERDA, reference check with Jesse |
| H2 Delivery | Pipeline | US number similar to NYSERDA, reference check with Jesse |
| H2 Delivery | Truck | US number similar to NYSERDA, reference check with Jesse |

Renewable Energy: Includes onshore wind, offshore wind, solar, and nuclear energy.

H2 Production: Primarily relies on PEM (Proton Exchange Membrane) electrolyzers to convert renewable energy into hydrogen.

Storage: Hydrogen can be stored in geological formations or tanks, providing flexibility and resilience to the supply chain.

H2 Delivery: Includes pipelines and trucks, facilitating the distribution of hydrogen across various nodes and to end users.

Example Results:

1. Hydrogen Cost Calculation: Total CAPEX, OPEX, and Levelized Costs

a. Total CAPEX (Capital Expenditure)

The total CAPEX is the upfront cost required to build and install hydrogen production facilities, including the cost of renewable energy plants, electrolyzers, storage units, and pipelines lines.

Total CAPEX= (CAPEXRE+CAPEXElectrolyzer+CAPEXStorage+CAPEXTransmission )

For RE, electrolyzer and storage units. Build costs are inputs which are the initial cost per unit capacity (e.g., dollars per kW or kg) for each type of technology is used. This can be derived from historical data, industry reports, or project-specific estimates. Zone Capacities is the total installed capacity in each zone, which is the direct output from the model, this is multiplied by the annualized build cost based on project recovery tme to obtain the total CAPEX for each technology.

- Formula:

- CAPEXRE​=Build CostRE​×Zone CapacitiesRE​

b. OPEX (Operational Expenditure)

OPEX represents the ongoing costs of operating and maintaining the hydrogen production system, including fuel costs, maintenance, and operational labor for each component (renewables, electrolyzers, storage, transmission).

Total OPEX= (OPEXRE+OPEXElectrolyzer+OPEXStorage+OPEXTransmission )

The OPEX is typically expressed as a percentage of the CAPEX, depending on the specific component.

c. Levelized Hydrogen Cost (LCOH)

The Levelized Cost of Hydrogen (LCOH) is the average cost per unit of hydrogen produced, accounting for all capital and operating expenses over the system's lifetime. It combines both CAPEX and OPEX, divided by the total amount of hydrogen produced over the project's operational life.

LCOH=∑(CAPEX+OPEX)Total Hydrogen Production (kg)LCOH=Total Hydrogen Production (kg)∑(CAPEX+OPEX)​

The LCOH helps compare different hydrogen production methods by providing a single cost metric.

2. Transmission Infrastructure

a. Transmission Line Buildout

The transmission infrastructure refers to the power lines required to transport renewable electricity from the generation sites to the electrolyzers. The cost of transmission lines depends on their length and capacity, and the power losses incurred during transmission.

Transmission Cost=∑(Length of Transmission Line (km)×Cost per km)Transmission Cost=∑(Length of Transmission Line (km)×Cost per km)

Transmission losses must also be accounted for:

Transmission Losses=Pinitial×(1−ηtransmission)Transmission Losses=Pinitial​×(1−ηtransmission​)

Where ηtransmissionηtransmission​ is the efficiency of the transmission system.

3. Buildout Map and Capacity (Renewables + Electrolyzers)

a. Renewable Energy Capacity

The capacity of renewable energy (RE) sources—such as solar, wind (onshore and offshore)—is an important factor in hydrogen production. The installed capacity is the maximum amount of electricity these sources can generate.

RE Capacity=∑(Installed CapacitySolar+Installed CapacityOnshore Wind+Installed CapacityOffshore Wind)RE Capacity=∑(Installed CapacitySolar​+Installed CapacityOnshore Wind​+Installed CapacityOffshore Wind​)

The capacity determines the total potential for renewable energy production, but actual production is dependent on the capacity factor.

b. Electrolyzer Capacity

The electrolyzer capacity dictates how much hydrogen can be produced from electricity. The total installed electrolyzer capacity across all zones is:

Electrolyzer Capacity=∑(Installed CapacityPEM Electrolyzer)Electrolyzer Capacity=∑(Installed CapacityPEM Electrolyzer​)

This capacity directly affects the hydrogen production rate, which is limited by both available electricity and electrolyzer capacity.

c. Buildout Map

The buildout map visualizes where renewable energy plants, electrolyzers, transmission lines, and storage units are located, along with their capacities. This helps optimize the system by reducing distances between components and ensuring an efficient supply of electricity to the electrolyzers.

4. Storage Buildout and Capacity

a. Storage Capacity

Energy storage is used to smooth fluctuations in renewable electricity generation. Storage technologies (e.g., batteries, hydrogen storage tanks) are critical for balancing supply and demand in the hydrogen production process. The total installed storage capacity is:

Storage Capacity=∑(Installed CapacityBattery+Installed CapacityHydrogen Storage)Storage Capacity=∑(Installed CapacityBattery​+Installed CapacityHydrogen Storage​)

Storage is typically measured in MWh for batteries or kg for hydrogen storage.

b. Storage Buildout Costs

The cost of building storage infrastructure includes both CAPEX and OPEX, similar to other system components:

Storage Buildout Cost=∑(CAPEXStorage+OPEXStorage)Storage Buildout Cost=∑(CAPEXStorage​+OPEXStorage​)

Storage systems enable the system to maintain continuous hydrogen production even during periods of low renewable generation.

5. Operating Dynamics for RE, Electrolyzers, Transmission, and Storage

a. Renewable Energy Operation

Renewable energy sources operate intermittently depending on resource availability (sunlight for solar, wind speed for wind farms). Their operation is governed by the installed capacity and capacity factor:

RE Operation (MWh)=Installed Capacity×Capacity Factor×8760 hours/yearRE Operation (MWh)=Installed Capacity×Capacity Factor×8760 hours/year

The actual energy output depends on the hourly or seasonal resource availability and weather patterns.

b. Electrolyzer Operation

Electrolyzers operate based on the availability of electricity from renewable sources and grid electricity (if available). Their operational efficiency and capacity determine hydrogen production:

Electrolyzer Operation (kg H2)=Electricity Input (MWh)×ηelectrolyzerHydrogen Energy Content (MWh/kg)Electrolyzer Operation (kg H2​)=Hydrogen Energy Content (MWh/kg)Electricity Input (MWh)×ηelectrolyzer​​

Where ηelectrolyzerηelectrolyzer​ is the electrolyzer efficiency, and hydrogen energy content is 33.33 kWh/kg.

c. Storage Operation

Storage systems store excess renewable energy or hydrogen and discharge it when required. Their operation depends on the state of charge or fill level and the round-trip efficiency:

Energy Discharged (MWh)=Energy Stored (MWh)×ηround-tripEnergy Discharged (MWh)=Energy Stored (MWh)×ηround-trip​

Where ηround-tripηround-trip​ is the storage system's efficiency during the charge-discharge cycle.

6. Levelized Cost of Electricity (LCOE) and Capacity Factors

a. Levelized Cost of Electricity (LCOE)

The Levelized Cost of Electricity (LCOE) is the cost per unit of electricity generated by renewable sources, considering CAPEX, OPEX, and total electricity production over the asset's lifetime.

LCOE=Total CAPEX+Total OPEXTotal Electricity Produced (MWh)LCOE=Total Electricity Produced (MWh)Total CAPEX+Total OPEX​

This provides a useful metric for comparing the cost-effectiveness of different renewable energy technologies.

b. Capacity Factor (CF)

The Capacity Factor measures the actual output of a power generation technology compared to its maximum possible output.

- Renewable Energy Capacity Factor:

CFrenewable=Total Renewable Production (MWh)Installed Capacity (MW)×8760 hours/yearCFrenewable​=Installed Capacity (MW)×8760 hours/yearTotal Renewable Production (MWh)​

- Electrolyzer Capacity Factor:

CFelectrolyzer=Total Electricity Consumed by Electrolyzers (MWh)Installed Electrolyzer Capacity (MW)×8760 hours/yearCFelectrolyzer​=Installed Electrolyzer Capacity (MW)×8760 hours/yearTotal Electricity Consumed by Electrolyzers (MWh)​

Capacity factors help in assessing the utilization of installed capacity and optimizing system performance.

Summary

This documentation provides an overview of the key concepts, formulas, and methodologies needed to derive essential results for hydrogen production systems. By calculating Total CAPEX, OPEX, LCOE, Capacity Factors, and other metrics, users can evaluate the technical and economic performance of hydrogen production facilities based on renewable energy sources and optimize the system for cost-effective, large-scale hydrogen production.