## HYPSTAT Open-sourcing Example Hydrogen Case Study Background

The case study examines the integration and distribution of hydrogen production and storage across various nodes, each with distinct characteristics and demand profiles. The nodes evaluated include a connection point, industrial site, city, inland area, suburb, and coastal area. Each node's capability to support renewable energy sources such as onshore wind, offshore wind, solar, and nuclear energy is assessed, along with their grid connectivity, export/import capacity, and storage capabilities. The case study is designed to showcase the modeling capability of Hydrogen Production, Storage, and Transmission Analysis Tool (HYPSTAT) model.

Case Design:

| Node | Descriptor | Demand | Onshore Wind | Solar | Offshore Wind | Nuclear | Grid | Export | Import | Geologic Storage | Tank Storage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Connection point | NO | YES (Good) | YES (Poor) | NO | NO | YES (Wholesale, lower) | YES | YES | NO | YES |
| B | Industrial site | YES | YES (Good) | YES (Poor) | NO | YES | YES (Fixed industrial) | NO | NO | NO | YES |
| C | City | YES | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| D | Inland | YES | YES (Poor) | YES (Good) | NO | NO | YES (Wholesale, higher) | NO | NO | YES (20 kT) | YES |
| E | Suburb, peaker plant | YES | NO | YES (Good) | NO | NO | YES (Fixed residential) | NO | NO | NO | YES |
| F | Coastal | YES | NO | YES (Good) | YES | NO | YES (Wholesale, higher) | NO | NO | NO | YES |

Node A (Connection Point): This node serves as a major junction in the hydrogen network, with no local demand but significant import and export capabilities. It is well-equipped with good onshore wind resources and modest solar potential. It connects to the wholesale grid at a lower capacity and has tank storage facilities.

Node B (Industrial Site): Characterized by a relatively large and flat demand profile, this site supports both onshore wind and solar (though with poor offshore wind potential). It has a fixed industrial grid connection, allowing it to manage stable hydrogen production and storage.

Node C (City): The city experiences medium demand, peaking in winter for heating and a consistent baseline for fleet vehicles. It lacks significant renewable energy resources and grid connectivity for hydrogen export or import.

Node D (Inland): With small demand primarily for heavy-duty fuel cell electric vehicles (HD FCEVs), this node has poor onshore wind but good solar potential. It connects to the wholesale grid at a higher capacity and has significant geologic storage (20 kT) alongside tank storage.

Node E (Suburb, Peaker Plant): This node has a large demand, especially in summer for power generation. It has good offshore wind potential and a fixed residential grid connection, though it lacks geological storage.

Node F (Coastal): The coastal node features medium demand for port operations with relatively flat consumption patterns. It harnesses good offshore wind resources and is connected to the wholesale grid at a higher capacity. Tank storage is available, but there is no geologic storage.

Technologies and Cost Reference:

| Type | Tech | Cost Reference |
| --- | --- | --- |
| Renewable | Onshore wind | NREL ATB moderate |
| Renewable | Offshore Wind | NREL ATB moderate |
| Renewable | Solar | NREL ATB moderate |
| Renewable | Nuclear | LCOE from H2@Scale |
| Renewable | Grid | NREL Cambium (Mid-case) for US national in hourly resolution (total_cost_enduse) |
| H2 production | PEM Electrolyzer | H2A |
| Storage | Geologic | US number similar to NYSERDA, reference check with Jesse |
| Storage | Tank | US number similar to NYSERDA, reference check with Jesse |
| H2 Delivery | Pipeline | US number similar to NYSERDA, reference check with Jesse |
| H2 Delivery | Truck | US number similar to NYSERDA, reference check with Jesse |

Renewable Energy: Includes onshore wind, offshore wind, solar, and nuclear energy, all of which are crucial for sustainable hydrogen production.

H2 Production: Primarily relies on PEM (Proton Exchange Membrane) Electrolyzers to convert renewable energy into hydrogen.

Storage: Hydrogen can be stored in geological formations or in tanks, providing flexibility and resilience to the supply chain.

H2 Delivery: Includes pipelines and trucks, facilitating the distribution of hydrogen across various nodes and to end users.

This structured evaluation aims to identify optimal configurations and strategies for hydrogen production, storage, and distribution, ensuring a robust and efficient hydrogen economy.

Example Results:

The results of the case study is

Hydrogen Cost (Total CAPEX, OPEX, levelized costs)

Transmission infrastructure

Buildout map and capacity  (RE + electrolyzers)

Storage buildout and capacity

Operating dynamics for RE, electrolyzers, transmission, and storage