Hydrogen Production, Storage, and Transmission Analysis Tool (HYPSTAT) Model Documentation

Joe Brauch, Yijin Li, Steven Percy, Jesse Cruce, Mark Ruth?, Laura Vimmerstedt?

This page can be deleted for final PDF preparation when the cover will be added.

Foreword

Body text is Times New Roman 12. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text.

Preface

Body text is Times New Roman 12. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text.

Acknowledgments

Body text is Times New Roman 12. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text.

List of Acronyms

AAA	Define abbreviations

BAT	Define acronyms

CAD	Define initialisms

Executive Summary

The Hydrogen Production, Storage, and Transmission Analysis Tool (HYPSTAT) is a modeling framework developed by the National Renewable Energy Laboratory (NREL) to support the analysis of hydrogen systems. HYPSTAT focuses on key components of hydrogen infrastructure, particularly electrolytic hydrogen production, hydrogen storage, and hydrogen transmission, as part of the transition to decarbonized energy systems

HYPSTAT operates as a supply-to-demand model, taking a fixed exogenous demand as input and optimizing the design and operation of the hydrogen system to meet that demand. It determines cost-optimal configurations based on specified technology options and system constraints. HYPSTAT formulates the hydrogen system as a linear or mixed-integer linear optimization problem using the Pyomo package in Python. The tool supports key processes such as input handling, model formulation, and solving. While basic output processing capabilities are under development, the primary focus remains on model formulation and optimization. HYPSTAT can be configured in two optimization modes: mixed-integer and fully linear. The mixed-integer mode captures non-linear pipeline costs, while the fully linear mode simplifies the formulation and reduces solve times.

HYPSTAT does not account for endogenous market dynamics such as fluctuating supply and demand conditions or hydrogen pricing. The demand is treated as fixed, and the results capture system costs but do not estimate hydrogen market prices directly. Additionally, the model is designed to simulate a single year at a time, making it unsuitable for capacity expansion analysis, although the input structure allows for easy execution of multiple year analyses to draw insights about long-term capacity growth.

To improve pipeline location results, HYPSTAT supports a two-step solve approach, first using coarser resolution and then refining with higher resolution. The tool also offers flexible time resolution, allowing users to down-sample input data for faster model solves.

HYPSTAT is designed to assist in evaluating system-level hydrogen infrastructure needs, offering insights into optimal system configurations, including the integration of renewable energy and hydrogen production systems. Its outputs can inform decision-making for policymakers, system planners, and energy developers, especially in areas focused on the transition to clean energy systems. While HYPSTAT provides valuable insights into hydrogen system operations and infrastructure needs, it has some limitations. These include its inability to capture dynamic market behaviors and the omission of non-electrolytic hydrogen production. Additionally, limited output processing capabilities are currently under development, suggesting further expansion of HYPSTAT’s functionality in future versions.

Table of Contents

Executive Summary	vii

1	Quick Start Guide	1

1.1	Designing your system	1

1.2	Formatting your inputs	1

1.3	Running the model	1

2	Detailed Documentation	2

2.1	Overview	2

2.1.1	Terminology	2

2.1.2	Fundamental units	3

2.2	Scope and Functionality	4

2.2.1	Electricity Generation Technologies	5

2.2.2	Hydrogen Production and Demand	6

2.2.3	Technology Costs	7

2.2.4	Imports and Exports	7

2.2.5	System Outputs	7

2.2.6	Recommended Scope	8

2.3	Workflow	9

2.3.1	Input Handling	9

2.3.2	Model Formulation	10

2.3.3	Solving the Model	10

2.3.4	Recommended Two-Step Solve Process	11

2.3.5	Writing Outputs	11

2.4	Inputs	11

2.4.1	YAML File	12

2.4.2	Individual Input Files	13

2.5	Outputs	17

2.5.1	Output Files	17

2.5.2	Suggested use of outputs	19

3	A Simple Case Study	21

3.1	Case Design	21

3.1.1	Nodes and Network	21

3.1.2	Technologies and Cost References	23

3.2	Potential Considerations for Results Processing	23

3.2.1	Hydrogen Cost Calculations: Total CAPEX, OPEX, and Levelized Costs	23

3.2.2	System Design	25

3.2.3	Operating Dynamics for RE, Electrolyzers, Transmission, and Storage	26

3.2.4	Levelized Cost of Electricity (LCOE) and Capacity Factors	26

Glossary	28

References	29

Appendix A: HYPSTAT Optimization Formulation	1

Demand Constraints	4

Import and Export Constraints	4

Storage Constraints	5

Link Flow Constraints	6

Production Constraints	7

Power Balance Constraint	8

Transmission and Storage Cost Constraints	9

3.3	Objective Function	10

To update this list, click on the list and press the F9 key. You can also right-click on the list and select Update Field. A dialogue box will prompt you to update the page numbers only or update the entire table. Delete these instructions when you are done with them.

List of Figures

Figure 1. Photo of a sunset (Arial 10 Bold)	2

Figure 2. Photo of winter (Arial 10 Bold)	5

To update this list, click on the list and press the F9 key. You can also right-click on the list and select Update Field. A dialogue box will prompt you to update the page numbers only or update the entire table. Delete these instructions when you are done with them.

List of Tables

Table 1. First Example Table (Arial 10 Bold)	2

Table 2. Second Example Table (Arial 10 Bold)	4

Table 3. Third Example Table (Arial 10 Bold)	6

Table A-1. Example Table in the Appendix (Arial 10 Bold)	10

To update this list, click on the list and press the F9 key. You can also right-click on the list and select Update Field. A dialogue box will prompt you to update the page numbers only or update the entire table. Delete these instructions when you are done with them.

Quick Start Guide

HYPSTAT is a supply-to-demand hydrogen system modeling tool developed by the National Renewable Energy Laboratory (NREL) to analyze hydrogen production, storage, and transmission infrastructure. It focuses on cost-optimization of hydrogen systems based on fixed exogenous demand, electrolytic hydrogen production, hydrogen storage, and transmission. HYPSTAT utilizes linear and mixed-integer linear optimization models, primarily formulated using the Pyomo package in Python. It is designed to simulate system-wide hydrogen infrastructure needs, offering insights into system configurations and helping policymakers and energy planners make informed decisions.

Designing your system

Hydrogen demand: Specify fixed exogenous hydrogen demand amounts and profile for a defined model time window.

Technology options: Select or define relevant hydrogen production (e.g., electrolyzers), storage, and transmission technologies. HYPSTAT supports renewable-powered electrolytic production but does not currently support non-electrolytic production methods.

Hydrogen system network: Define hydrogen network, design node, link distances, and physical constraints for the network

| Technology/input | Suggested scope |
| --- | --- |
| Nodes | 5-15 nodes |
| Links | 10-30 links |
| Renewable production | 3-5 technologies with 1-3 tranches per technology and node |
| Hydrogen storage | 2-4 technologies |
| Time period | Coarse resolution: ~daily Fine resolution: 1-4 hourly |
| Time window | 1 year |

Constraints: Establish system constraints such as import/export limits, technology-specific capacities for each node.

Technology cost: Define economic parameters like CAPEX and OPEX, along with financial parameters such as WACC.

Transmission (pipeline or tank-based) location & size: For accurate pipeline location, use the two-step solve approach, which refines pipeline locations after an initial coarser resolution solve.

Formatting your inputs

YAML Configuration File:

Input your scenario configuration into a YAML file.

Specify paths to input files for technology costs, generation profiles, and demand data.

Technology Costs:

Define annual CAPEX, fixed OPEX, and variable OPEX for each technology.

Costs are scaled based on the model time window.

Time Resolution:

Define time-based inputs like renewable generation profiles, hydrogen demand, and storage dynamics.

The model supports down-sampling of input resolution but cannot up-sample.

Pipeline Inputs:

For mixed-integer optimization mode, input binary variables to optimize pipeline locations.

For fully linear optimization mode, input fixed pipeline costs.

Running the model

Load Inputs:

Use the HYPSTAT class to load your inputs from the YAML configuration file using the load_inputs method.

Model Formulation:

Call load_model and load_constraints methods to set up the optimization problem.

Define constraints related to generation, production, storage, and transmission technologies.

Solve the Model:

Use the solve_model method with the desired solver (GLPK or Gurobi recommended).

Pass the optimize_pipelines parameter to select between mixed-integer (pipeline optimization) or fully linear (fixed pipeline locations) modes.

Two-Step Solve Approach:

For accurate pipeline location results, run the model twice: first with coarser resolution (daily) and then refine with finer resolution (e.g., 1-2 hourly) using fully linear optimization for accurate dynamic results.

Write Outputs:

Use the write_outputs method to save model results to CSV files.

Specify output paths and handle overwriting options asnecessary. ---

Detailed Documentation

Overview

The Hydrogen Production, Storage, and Transmission Analysis Tool (HYPSTAT) is a supply-to-demand hydrogen system modeling tool developed at the National Renewable Energy Laboratory (NREL). HYPSTAT is used to represent system-wide hydrogen infrastructure needs,  particularly for transitioning to decarbonized energy systems with a focus on electrolytic hydrogen production along with hydrogen storage and transmission.

HYPSTAT is a supply-to-demand model: it is fed exogenously a fixed demand and determines the cost-optimal system design and operation to meet that demand based on specified technology options and system constraints. It models a single year at a time; that is, it is not a capacity expansion model. However, HYPSTAT inputs are set up to easily run separate, individual analyses for several different years to infer insights about capacity expansion.

HYPSTAT also does not include endogenous market dynamics around supply and demand. The demand is fixed, and systems defined by the results have associated costs but do not specifically estimate hydrogen market prices.

HYPSTAT formulates the specified hydrogen system as a linear or mixed-integer linear optimization using the Pyomo package in Python. It also includes code for input processing, limited output processing capabilities are in development.

Terminology

Within the HYPSTAT model and this documentation, we use certain terminology to refer to specific things like electricity generation or hydrogen production. These often have standard abbreviations in our code base and naming conventions. In general, we use “generation” (gen) to refer to electricity generation, “production” (prod) to refer to hydrogen production, “storage” (stor) to refer to physical hydrogen storage, and “transmission” (trans or transmission) to refer to physical hydrogen transport. In these categories, we refer to “technologies” (tech) for different types of technology, for example:

Wind, solar or grid electricity would be three types of generation technology; and

Truck transmission and pipeline transmission would be two types of transmission technology.

Different technology types have names which are either flexible via user inputs or fixed for certain uses. Note that for simplicity, we use electricity generation to refer to any provider of electricity, even if that technology is not technically generating the electricity—e.g., grid electricity. HYPSTAT currently supports generic technology types for generation and storage but only considers pre-specified technology types for production and transmission (see Scope and Functionality).

See below for other specific terminology and definitions in the context of the HYPSTAT model.

Node: a representation of a location in the model which can contain electricity generation technology, hydrogen production technology, and hydrogen storage technology.

Link: a connection between two nodes over which hydrogen can be transported.

Capacity: the amount or size of a certain technology that is built at a node.

Time period or simply period: the individual unit of time for which the model considers dynamics of the hydrogen system—hydrogen production, electricity production, hydrogen transmission, storage injections and withdrawals, etc. (e.g., hourly).

Model window: the entire time length for which the model captures the hydrogen system (e.g., a full year).

Storage level: the amount of hydrogen that is stored in a hydrogen storage resource at a given time, typically measured in kilograms (kg) of H2.

Storage charging: injecting hydrogen into hydrogen storage (similar to charging a battery), typically measured in kg of H₂ per time period.

Storage discharging: withdrawing hydrogen from hydrogen storage (similar to discharging or using energy from a battery), typically measured in kg of H₂ per time period.

CAPEX: overnight capital expenses to build a given technology, including renewable energy systems, electrolyzers, storage, and transmission technologies.

Fixed OPEX: annual operating expenses to operate a given technology which are fixed regardless of the amount of use of or production from that technology.

Variable OPEX: operating expenses incurred for using a given technology, calculated per unit of production or use (e.g., per kilogram of hydrogen stored or transmitted).

Unserved hydrogen: hydrogen demand which is not met in the model. Unserved hydrogen is essentially used as a tool to avoid infeasibility of systems—it is given a very high cost, thus is only used if demand is impossible to meet.

Overserved hydrogen: Hydrogen that is produced but not required to meet demand or exports. This is primarily a modeling feature and is assigned a cost (e.g., an approximate cost of storing excess hydrogen) to help prevent unrealistic outputs, such as excessive hydrogen production driven by policy incentives.

HYPSTAT also has two options for model formulation and solving: a mixed-integer optimization mode, and a fully linear optimization mode. The mixed-integer mode is a better representation of the hydrogen system and optimizes pipeline locations, but it is much more complex and solves more slowly. The fully linear mode is a more approximate representation of the system which solves faster but does not accurately optimize for pipeline locations and requires an input for where pipelines will be constructed. For more detail, see Solving the Model.

Fundamental units

HYPSTAT treats inputs and variables as having standardized units, shown below for different kinds of inputs and variables. A user could set up a HYPSTAT model such that it uses different units, but they must be consistent across the categories below to avoid errors. We strongly recommend against this to avoid errors. (These units are also noted in many of the input files.)

In particular, note that the only conversion factor that is a user-specified input is a hydrogen production efficiency, designed to be a figure in kWh electricity/kg hydrogen.

Currency: dollars ($)

Electricity generation and hydrogen production (electrolyzer) capacity: kilowatts (kW)

Electricity generation amounts: kilowatt-hours (kWh)

Hydrogen storage capacity and production, storage level, and transmission amounts: kilograms (kg) of H2

Hydrogen storage charge and discharge: kg/hr (in inputs) or kg/period (in code and model outputs)

Hydrogen transmission flow amounts: kg/hr (in inputs) or kg/period (in code and model outputs)

Scope and Functionality

HYPSTAT is designed to capture hydrogen systems defined by sets of nodes and potential linkages. Each node can accommodate hydrogen production, demand, and storage, while each link can represent potential hydrogen transmission between nodes.. HYPSTAT primarily focuses on electrolytic hydrogen production from renewable energy, so production potential at nodes is built around representation of renewable energy and electrolysis. This can be adapted for other technologies, such as grid-powered electrolysis. Note that non-electrolytic hydrogen production is not currently supported in HYPSTAT.

HYPSTAT allows for flexible representation of any number of electricity generation and hydrogen storage technologies. For example, a HYPSTAT model could include:

Electricity generation technologies:

Land-based wind

Solar PV

Offshore wind

Grid electricity

Storage technologies:

Above ground tank storage

Buried pipe storage

Geologic salt cavern storage

The names of these technologies are set via user inputs. We recommended names with no spaces. Note that electricity generation technologies and storage technologies cannot have the same name, nor can any technology be named ‘Electrolyzer’ or ‘Pipelines’, as this will cause errors in how the model reads and processes inputs.

Currently, HYPSTAT only allows for two transmission technologies: pipeline transmission and tank-based transmission via trucks. (Generalization of additional transmission technologies may be added in the future.) These are referred to as ‘Pipelines’ and ‘Trucks’ in the model inputs and code base. HYPSTAT also only allows one type of hydrogen production technology, electrolysis, which is referred to via the technology name ‘Electrolyzer’ in model inputs. (additional hydrogen production technologies may be added.)

The technologies are set up in HYPSTAT to perform their inherent functionality and are further defined by costs and constraints which are user inputs. Further details are described below.

Table 1. Technology options in HYPSTAT

| Technology | Description | Allows for subtypes? | Cost inputs | Constraints | Incentives |
| --- | --- | --- | --- | --- | --- |
| Electricity generation (gen) | Generates or provides electricity which can be used to produce hydrogen | Yes | CAPEX, fixed OPEX, time-dependent variable OPEX | Capacity build constaint and generation profiles which specify how much is electricity can be produced in each time period (e.g., to represent renewable profiles) | Per kWh credits which can be applied to either potential generation (e.g., representative of the ITC) or actual generation (e.g., representative of the PTC) |
| Hydrogen production (prod) | Uses electricity to produce hydrogen | No | CAPEX and fixed OPEX, (variable OPEX may be added) | Produces hydrogen based on the amount of electricity generated and used at that node and a user-specified efficiency | None |
| Hydrogen storage (stor) | Stores hydrogen over time periods | Yes | CAPEX, fixed OPEX, and variable OPEX for charging only | User inputs on constraints for how quickly storage can be charged or discharged as fractions of capacity. Adheres to conservation of mass. | None |
| Pipeline transmission (Pipelines) | Moves hydrogen from one point to another | No | CAPEX is based on pipeline size and includes a fixed CAPEX adder for whether or not the pipeline is built. Fixed OPEX is calculated as a fraction of CAPEX, while variable OPEX applies to pipeline flow in either direction. | Allows user input for maximum pipeline size for each link | None |
| Tank-based transmission via trucks (Truck) | Moves hydrogen from one point to another, designed to represent truck, rail, barge, etc. transmission | No | Variable OPEX for flow in either direction, i.e., designed to represent a levelized cost of transmission | Allows user input for maximum flow for each link | None |

Electricity Generation Technologies

Electricity generation technologies produce electricity based on input generation profiles for each time period. The profiles provide a maximum potential generation in each period per unit of capacity built—essentially, a capacity factor for that time period. This is primarily meant to represent renewable resources for which profiles can be obtained using a tool like the System Advisor Model or NREL’s Renewable Energy Potential (reV) Model. For each technology at each node, a technology can have one or more supply curve profile tranches which each contain a potential profile and a capacity limit. Tranches can be used to represent differing levels of resources within the area represented by a node, i.e., to capture a renewable supply curve. For example, perhaps a region has a small area of good land-based wind resource and a larger area of worse land-based wind resource. This could be represented in HYPSTAT with two tranches for land-based wind at that node, each with different generation profiles and capacity limits.

While electricity generation technologies are designed around renewables, they can also be used to represent other electricity sources such as the grid electricity using the variable operating expense inputs. HYPSTAT supports profiles for operating expenses on a per unit (e.g., kWh) basis for each time period, which can be used to represent constant or varying (e.g., wholesale) grid prices. This will require a profile and capacity limit for the grid and CAPEX and fixed OPEX inputs—we recommend setting these to zero and providing a profile of all “1s” such that the model could potentially buy as much electricity from the grid as it would like. The capacity limit can be used to place a cap on the amount of grid electricity that could be purchased (e.g., from a transmission limit), or the profile could be used if this varies with time. CAPEX and fixed OPEX for the grid could be used to represent those costs for installing a grid connection, if desired.

Based on the profiles and capacity built, HYPSTAT determines the maximum electricity generation or generation potential and decides how much to allocate to hydrogen production and how much to curtail. The decision on curtailment also considers how much grid electricity to purchase, as variable OPEX—representing grid electricity costs—is only incurred for electricity that is actually used.

Electricity transmission between nodes is not supported in HYPSTAT.

Hydrogen Production and Demand

Hydrogen production technology can utilize all electricity generated at a given node and only that node, regardless of generation technology. This allows for HYSPTAT to consider hybridization, e.g., a combined wind and solar to hydrogen plant, and optimize for the sizes of those technologies. HYPSTAT does not allow for specific electricity generation going to specific hydrogen production technologies (e.g., to represent nuclear electricity paired with solid oxide electrolysis), but this feature may be added in the future.

HYPSTAT is designed for strict demand enforcement—i.e., hydrogen demand must be completely met in the period during which the demand occurs.

Technology Costs

CAPEX and fixed OPEX for all technologies are specified as a single, per-unit overnight CAPEX and per-unit annual OPEX cost (e.g., $/kW and $/kW-year for electrolyzers). Because it is a linear model, HYPSTAT does not allow for non-linear CAPEX relationships or varying CAPEX values at different sizes to account for economies of scale. The exception to this is pipeline transmission: because pipeline costs are highly non-linear with scale, we have developed an approach using a linear approximation of costs that can include a non-zero y-intercept. This represents pipelines as having a single, fixed CAPEX that is incurred if the pipeline is constructed, regardless of size, plus an additional CAPEX adder that is dependent on the pipeline scale. Note that we consider pipeline scale based on flow (e.g., kg/hr of hydrogen), not on diameter since diameter scales non-linearly with volume of flow. We find this to be a good approximation for pipeline costs:

Figure 1. Approximation of pipeline costs used in HYPSTAT

Figure notes, if needed, are Arial 9.

HYPSTAT does not currently support variable OPEX for hydrogen production, since variable OPEX on a per kg basis is inevitable given fixed production to meet the given hydrogen demand (i.e., this variable OPEX would not affect the optimization results). We recommend that the user adds in or considers variable OPEX for hydrogen production in output processing steps for analyzing results.

Imports and Exports

HYPSTAT also allows for imports and exports in limited capacities, to allow for flexibility in a hydrogen system. Imports and exports are specified by volume limits: i.e., a maximum amount of allowed imports or minimum amount of exports that must be met. By default, there are no costs or values associated with imports and exports in the objective function, though these could be added to the objective function code by the user if desired. The user can specify which nodes (if any) imports and/or exports occur from. Note that import capabilities have been rigorously tested; export capabilities are based on the same dynamics as imports but have not been rigorously tested.

System Outputs

Based on these inputs, HYPSTAT sets up an optimization problem with variables which capture the following key design and operation specifications for a system to meet the specified demand:

System design:

Optimal capacity for electricity generation, hydrogen production, and hydrogen storage technologies at each node.

Optimal links for pipelines and the optimal size of each pipeline.

System operation:

Electricity generation and curtailment for each technology at each node in each time period

Hydrogen production at each node in each time period

Injections and withdrawals from storage at each node in each time period

The amount of stored hydrogen (storage level) at each node in each time period

Hydrogen flows via pipeline and via truck transmission for each link in each time period

The amounts of imports and exports at each node in each time period

The extent to which demand is met at each node in each time period (to verify if input demand is fully satisfied or to track when flexible demands are met).

If there is unserved hydrogen (i.e., demand that is impossible to meet) or excess hydrogen produced at any node in any time period

Recommended Scope

HYPSTAT models the hydrogen system at a time resolution which can be flexibly specified by the user. While this is flexible, HYPSTAT was designed to be modeled at a granular level in order to capture intra-year storage dynamics. HYPSTAT is also designed to model systems for a full year, but it can be set up to run for shorter model time windows (e.g., for quick solves and testing). In theory, HYPSTAT could be set up to run periods longer than a year, but this has not been tested.

While HYPSTAT is inherently flexible for the scope and scale of the systems it represents, we recommend limiting the scope to roughly these ranges for a single HYPSTAT run. However, multiple runs can be submitted either in parallel or consecutively for a comprehensive study of the desired scenarios:

| Technology/input | Suggested scope |
| --- | --- |
| Nodes | 5-15 nodes |
| Links | 10-30 links |
| Renewable production | 3-5 technologies with 1-3 tranches per technology and node |
| Hydrogen storage | 2-4 technologies |
| Time period | Coarse resolution: ~daily Fine resolution: 1-4 hourly |
| Time window | 1 year |

In our experience, models formulated within these bounds solve on the scale of hours or less using a high-performance computing system and a good solver (e.g., Gurobi). HYPSTAT could easily represent a smaller system, which may solve even faster. HYPSTAT could represent a larger system, but this has the potential to create very long and impractical solve times. Note that there are also many inherent design parameters of the system that can be independent of system size that may affect solve times (often related to the ease of meeting demand: for example, a system with significant flexible imports typically solves much faster than the same system when imports are restricted). Depending on these factors, a system in the above size/scope ranges could solve much faster or slower than expected.

Workflow

From a high level, the HYPSTAT code base works in three primary steps: input handling/collection, model formulation/solving, and output writing. The code base is primarily contained in the file ‘model_object.py’, with some helper functions imported from ‘model_functions.py’. These steps are organized into a single class ‘HYPSTAT’; an instance of the HYPSTAT class is used to load and run the model.

HYPSTAT’s default way of solving a problem involves a two-step process which loads and solves the model twice (more description below). In this case, the output writing step is only performed after the second solve.

Input Handling

When creating a HYPSTAT class, the user specifies a file path for a YAML file that contains the controls and inputs (or paths to various input files) for the scenario being modeled. See the Inputs section below for more detail. The user calls the ‘load_inputs’ method of the class to read in the proper inputs from the YAML and relevant files and process them into appropriate inputs for HYPSTAT. Some of the key processing steps that occur are:

Input values for CAPEX are annuitized using WACC and recovery times (also specified in the inputs) and combined with fixed OPEX to develop annual costs that are incurred per unit of capacity for each relevant technology.

The electricity generation and hydrogen storage technologies can be specified in the YAML file, but if they are not, the model infers these technologies based on the technologies listed in the relevant cost input files.

HYPSTAT calculates appropriate OPEX for pipeline and truck transmission for each link based on input costs and link distances.

Relevant quantities (annual costs, import and export limits) are scaled based on the length of the model window (for runs more or less than one year: e.g., for a 6-month model window, technologies should only incur half of their annual costs in the objective function).

HYPSTAT updates the time resolution of relevant time-based parameters (e.g., renewable generation profiles, hydrogen demand) if the model resolution does not match the input resolution. Note: the model cannot up sample input data, but it can down sample them. This can be useful, e.g., to have inputs specified hourly but easy ability to change the model to coarser resolution (e.g., 2-hourly) to achieve a faster solve time.

If relevant, pipeline locations are restricted based on results from a previous model iteration (see Recommended Two-Step Solve below).

Model Formulation

Model formulation is broken into two steps in HYPSTAT. First, the model loads index sets, parameters, and variables into the Pyomo optimization formulation via the load_model method. Next, HYPSTAT formulates the optimization problem with appropriate constraints and the objective function via the load_constraints method. From a high level, the constraints define the behavior of generation, production, storage, and transmission technologies based on physical principles, input constraints, and mass balances. The constraints also ensure that hydrogen demand is met, either rigidly or flexibly. The objective function represents the costs incurred by the system design and operation and is minimized. See the detailed model formulation for more information on the design and purpose of specific constraints and the objective function.

Solving the Model

The model solves via the solve_model method. This method is passed a string identifier for the solver which determines which solver Pyomo tries to use to solve the optimization problem. Currently, only GLPK and Gurobi are supported, but we encourage users to simply program their own lines or edit how the Pyomo SolverFactory object is created to use the solver of their choice.

A key parameter which is passed to the load_inputs, load_model, load_constraints, and solve_model methods is the boolean parameter ‘optimize_pipelines’. This refers to whether or not the locations of pipelines are specifically optimized in the model from options specified in the inputs; or whether pipeline locations are fixed going into the model solve. Note that this only refers the locations of pipelines, i.e., where pipelines are and aren’t built—the sizes of the pipelines that are constructed are always optimized in HYPSTAT.

If optimize_pipelines is set to True, HYPSTAT is formulated in its mixed-integer mode with a set of binary variables which represent whether or not a pipeline exists at each link. These binary variables are used to better capture the non-linearity of pipeline capital costs. However, they can complicate the model solve significantly and lead to much longer solve times.

If optimize_pipelines is set to False, HYPSTAT is formulated in its fully linear mode without these binary variables and every link incurs the fixed capital expense for building a pipeline there, regardless of whether a pipeline exists. Because of this, this cost is unavoidable and becomes a fixed adder on the objective function and has no impact on the optimization. (Note that this makes the value of the optimization function not accurately representative of total annual system costs in these cases.) Pipelines effectively only incur linearly-scaling capital and fixed operating costs from the variable size component. If pipeline locations are not properly constrained, this can result in impractically small pipelines which would be very expensive in reality, though HYPSTAT sees only a small fraction of their capital costs. However, the avoidance of binary variables allows for a fully linear formulation of the optimization problem which typically solves much faster. This mode is recommended if the user has a good idea of where pipelines should be built. It could also be used iteratively by successively restricting pipeline locations in the inputs where the optimal size identified by the model is below a certain threshold.

Recommended Two-Step Solve Process

For scenarios where the optimal location of pipelines is an important question, we have developed a recommended methodology and streamlined approach in HYPSTAT to address this while attempting to reduce solve times. We recommend loading and solving the model twice: first using a mixed-integer optimization with optimize_pipelines set to True but with a coarser time resolution—e.g., daily—then using a fully linear optimization with optimize_pipelines set to False with a finer time resolution, e.g., 1-2 hourly. The second optimization utilizes the locations of pipelines identified in the first optimization.

We find this approach to be effective for several reasons. Model solve time tends to increase dramatically and in a non-linear way with model complexity: e.g., making the time resolution twice as fine (roughly doubling the size of the model) typically results in much greater than 2x solve times. So, by simplifying the first optimization to a ~daily level, we achieve a 12-24x reduction in model size which dramatically reduces solve times. On an HPC system, these systems usually solve in a matter of minutes even when formulated with binary variables. This coarser time resolution is acceptable for setting the location of pipelines, which don’t typically depend on intricacies of intra-day system dynamics. Then, using these locations, every other variable is reoptimized at a fine time resolution to properly capture detailed dynamics of production, flow, and storage. We have done brief tests of standalone mixed-integer optimizations and this two-step approach at the same time resolution and find both minimal differences between pipeline locations, at many resolutions, and minimal differences in overall system results between hi-res mixed-integer optimizations and this simplified two-step approach.

The approach is coded into the model directly via the two_step_solve method. This method runs both iterations and retrieves the locations of pipelines using the Pipeline_Exists model variable between the first and second solve. These are passed to the second model solve during input loading via the Pipeline_Exists parameter of the load_inputs method. When this parameter is not None (its default), it overrides pipeline inputs to disallow any pipeline links that do not exist in the Pipeline_Exists parameter. (This parameter could be used outside of the two-step solve strategy as a Pandas Series with pipeline links for the index and values of 0 or 1 for if a pipeline is not constructed or constructed, if the user wanted to control pipeline locations for a solve with optimize_pipelines set to False without modifying underlying input parameters.)

Writing Outputs

After solving, the optimal result is contained in the Pyomo model object within the HYPSTAT object. These outputs can be written to a series of CSV files (described in the Outputs section) with the write_outputs method, which takes a path to a folder as an argument. If that folder does not exist, it will create it; if the folder does exist, HYPSTAT will ask the user if the results should overwrite contents in the folder, be written to a new directory specified by the user, or not written to any files.

Inputs

HYPSTAT inputs are contained in a YAML file with scenario information and a set of files containing detailed inputs for costs, demand and supply profiles, and network design. We recommend organizing the input files as shown in the Sample Case Study or similar, but ultimately they can be organized in any way as long as the file paths are correctly specified in the YAML file.

YAML File

A HYPSTAT scenario is ultimately defined by a YAML file that is provided to the HYPSTAT model class to load the scenario. The YAML file specifies high level scenario parameters and contains file paths to other individual files that make up scenario inputs.

The YAML file should contain the following parameters:

analysis_year (int): the year to be modeled. Used for ease of setting up analyses with multiple years (inputs can be formatted to capture multiple years instead of needing entire sets of inputs for each year).

nodes (list): the list of nodes to be considered.

max_imports_ratio (float): the fraction of the total hydrogen demand which can be met with imports

import_nodes (list): the list of nodes which are allowed to import hydrogen

min_exports_ratio (float): the minimum amount of hydrogen that must be exported, as a fraction of total demand (exports are additive to demand and not considered part of demand, i.e., they can be met flexibly at any time)

export_nodes (list): the list of nodes which are allowed to export hydrogen

overserved_cost (int or float): the cost per kg of overserved hydrogen

unserved_cost (int or float): the cost per kg of unserved hydrogen

start (str in the format ‘D MMM’): the start date for the analysis window

end (str in the format 'D MMM’): the end date for the analysis window

coarse_resolution (int): the time resolution in hours for a run in which pipelines are optimized with binary variables

fine_resolution (int): the time resolution in hours for a run in which pipeline locations are not optimized and the model is fully linear

Two particular parameters of note are unserved_cost and overserved_cost. We recommend giving unserved hydrogen a very high cost (e.g., $1,000/kg). This is essentially a modeling tool to prevent infeasibility in the model, but at such a cost unserved hydrogen should never be used by the model to close the mass balance unless it is impossible otherwise. Overserved hydrogen is also given a cost—for example, an approximation of the cost of storing excess hydrogen—which should in almost all cases prevent the model from using it, since it should have the option to simply curtail electricity for free. For advanced use of the model, the costs for unserved and overserved hydrogen could be adjusted (or even made negative to give a value to overserved hydrogen) to test system dynamics such as how much excess capacity might be worth building, or how expensive it is to meet certain hydrogen demands.

The YAML file may also contain:

gen_techs (list): the list of electricity generation technologies to be considered

stor_techs( list): the list of hydrogen storage technologies to be considered

If these are not included in the YAML file, they will be inferred from cost input files.

Lastly, the YAML contains the file or folder paths for the other necessary inputs, specified in the next section.

Individual Input Files

The majority of inputs are noted in a set of files, described below, which the YAML file references. Note that many files (e.g., for cost inputs) contain a column for year. HYPSTAT can only solve for one year at a time, but the year column is included so that scenario analyses can be designed and run for several years with only one set of input files.

Hydrogen demand

Demand is specified in a CSV file. Use the ‘DemandFile’ option for this in the YAML file. The demand file should contain columns for the node, time period, and hydrogen demand in that period, as shown below. All nodes and time periods must have a corresponding row, even if there is no demand—any omission will cause an indexing error. Note about period formatting

| Node | Period | h2_demand[kg/hr] |
| --- | --- | --- |
| A | D1 | 0 |
| A | D2 | 0 |
| … | … | … |
| B | D1 | 100 |
| B | D2 | 150 |
| … | … | … |

Generation profiles

Generation profiles for electricity generation technologies are input as a directory containing individual files for each technology, location, and year. The directory is specified with the ‘ProfilesDir’ option. The directory should contain a file for each allowed node, technology, and year option. Note that this is currently how allowed technology options are set in HYPSTAT. If a node cannot build a certain technology, it should not have a file in the profile directory. These files must be named ‘<tech>_Node_<node name>_<year>.csv’ (<> indicates a variable for different techs, nodes, and years). Each file must be formatted to contain the 8760 production profile of kWh of generation in each hour per kW of capacity. The columns are used to specify multiple tranches of a technology/resource at a single zone. Rows should correspond to each hour and column headers should note the maximum available capacity in that tranche in kW. (Values in the index column are ultimately ignored.)

|  | 140000 | 75000 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 0.43 | 0.15 |
| 2 | 0.77 | 0.64 |
| 3 | 1 | 0.93 |
| … | … | … |
| 8759 | 0.2 | 0 |

Network file

The network file is one of the more complex input files, specified with the ‘NetworkFile’ field. It specifies potential links between nodes, which transmission technologies are allowed, maximum capacities for each link and technology, and distances between links for each transmission technology. It should be formatted as shown below. Note that the link distances here are only used to calculate costs based on per-km cost inputs for truck and pipeline transmission. This is to facilitate easy set-up with actual network distances. However, distances can also be used to scale transmission costs for different links (e.g., if a particular pipeline link is more expensive due to difficult terrain, the user could artificially increase its pipeline distance).

| Link | Start node | End node | Pipeline allowed | Truck allowed | Pipeline distance [km] | Truck distance [km] | Pipeline max capacity [kg/hr] | Truck max capacity [kg/hr] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A to B | A | B | Y | Y | 100 | 120 | 500000 | 4166.667 |

Storage constraint files

Storage constraints and capacity limits are captured in two files, specified by the ‘StorCapacityFile’ and ‘StorLimitsFile’ field. They should be formatted as shown below. StorageCapacityFiles contains capacity limits for each storage technology and each zone, while ‘StorLimitsFile’ contains total capacity constraints for the entire system and limits for how much hydrogen can be charged or discharged per hour, as a fraction of working capacity. Capacity constraints are in kg of working capacity. To not impose a limit on a given capacity or constraint, use ‘inf’.

StorCapacityFile

| Node | A | B | … |
| --- | --- | --- | --- |
| Geologic | 0 | 2000 | … |
| Tank | inf | 0 | … |
| … | … | … | … |

StorLimitsFile

| Storage Tech | Capacity | Charge limit | Discharge limit |
| --- | --- | --- | --- |
| Geologic | 8000 | 0.001 | 0.004 |
| Tank | inf | inf | inf |
| … | … | … | … |

Electricity generation, hydrogen production, and hydrogen storage cost files

The costs for each of these technologies are contained in files specified in the ‘GenCostFile, ‘StorCostFile’, and ‘ProdCostFile’ fields. They should be formatted as shown below. HYPSTAT supports different cost inputs for different nodes, or the node column can simply read ‘All’ to be applied to all nodes. If a cost is specified for ‘All’ nodes and then also for any number of specific nodes, the cost for ‘All’ will be applied to any non-specified nodes and the specific nodal costs in the file will be used for those nodes only.

GenCostFile

| Year | Node | Tech | CAPEX ($/kW) | Fixed OPEX ($/kW-yr) | Variable OPEX ($/kWh) |
| --- | --- | --- | --- | --- | --- |
| 2030 | All | Solar_PV | 400 | 15 | 0 |

ProdCostFile

| Year | Node | Tech | CAPEX ($/kW) | Fixed OPEX ($/kW-yr) | Efficiency (kWh/kg) |
| --- | --- | --- | --- | --- | --- |
| 2030 | All | Electrolyzer | 1000 | 20 | 55 |

StorCostFile

| Year | Node | Tech | CAPEX ($/kg) | Fixed OPEX ($/kg-yr) | Variable OPEX ($/kg-charged) |
| --- | --- | --- | --- | --- | --- |
| 2030 | All | Salt_cavern | 30 | 0 | 0.06 |

Time-variable OPEX for electricity generation (optional)

If desired, a file or files can be provided for time-variable OPEX for specified electricity generation technologies (e.g., grid electricity). They should be specified under the ‘GenCostProfileFile’ field and then specified using the specific generation technology that they refer to. If files are provided for a given technology, these time-variable OPEX will override the OPEX noted in the ‘GenCostFile’. Files should be formatted as shown below, with 8760 data rows corresponding to each hour of the year. (The actual values of the index column are disregarded.)

|  | Price ($/kWh) |
| --- | --- |
| 0 | 0.02 |
| 1 | 0.03 |
| 2 | 0.77 |
| … | … |
| 8759 | 0 |

Hydrogen transmission cost files

The costs for pipelines and trucks are contained in the file specified in the ‘TransCostFile’ field. For now, the inputs must be referred to as ‘Pipeline’ and ‘Truck’ and do not support alternative costs for different links. (We recommend using scaled link distances to represent different costs for different links.) The costs are treated differently for pipelines and trucks but are considered in the same file, so they should be formatted as shown below. Most of the fields are irrelevant for trucks and should read ‘N/A’; the LCOT field is irrelevant for pipelines and should read ‘N/A’.

| Year | Tech | Fixed CAPEX ($/km) | Capacity CAPEX ($/(kg/hr)/km) | Fixed OPEX (fraction of CAPEX/yr) | Variable OPEX ($/kg/100-km) | LCOT ($/kg/100-km) |
| --- | --- | --- | --- | --- | --- | --- |
| 2030 | Pipeline | 1000000 | 10 | 0.01 | 0.06 | N/A |
| … | … | … | … | … | … | … |
| 2030 | Truck | N/A | N/A | N/A | N/A | 0.8 |
| … | … | … | … | … | … | … |

Financials

The field ‘FinancialsFile’ specifies a file which contains recovery times and weighted average costs of capital (WACCs) for each technology. This includes all electricity generation technologies, hydrogen storage technologies, hydrogen production, and pipeline and tank-based transmission. WACC should be input as a floating-point decimal, not a percentage (e.g., for a 10% WACC enter 0.1). Hydrogen production and pipeline transmission must be referred to as ‘Electrolyzers’ and ‘Pipelines’, and the file should be formatted as shown below.

| Tech | Year | Recovery_time | WACC |
| --- | --- | --- | --- |
| Electrolyzers | 2030 | 30 | 0.08 |
| Pipelines | 2030 | 25 | 0.1 |
| Solar | 2030 | 20 | 0.06 |
| … | … | … | … |
| Salt cavern | 2030 | 25 | 0.1 |
| … | … | … | … |

Incentives

Levelized electricity generation incentives, e.g., a production or investment tax credit, can be captured in HYPSTAT and are input in a file specified by the field ‘IncentiveFile’. It can include an ITC credit (applied to used and curtailed electricity) and a PTC credit (applied to only used electricity). This is for a levelized representation of the ITC; the ITC could also be represented by adjusting the CAPEX or relevant financial parameters. Note that every electricity generation technology must be included, even if the incentives are both 0. It should be formatted as shown below.

| Year | Tech | ITC ($/kWh) | PTC ($/kWh) |
| --- | --- | --- | --- |
| 2030 | Solar | 0 | 0.02 |
| 2030 | Offshore_Wind | 0.03 | 0 |
| 2030 | Grid | 0 | 0 |
| … | … | … | … |

Outputs

HYPSTAT outputs show optimal system design and operation for the specified scenario. These can be used to infer insights about optimal hydrogen systems and estimate cost contributions of different components like electricity generation, transmission, and storage.

Output Files

HYPSTAT collects the optimal values of model variables and reports them in a series of output files. For ease of processing and debugging, certain input parameters are also collected and written to output files. The output files are described below:

Files detailing input parameters

| File name | Description |
| --- | --- |
| build_cost.csv | Shows the resulting annuitized, scaled costs per unit of capacity for each technology and node that were determined based on CAPEX, fixed OPEX, and financing inputs. |
| incentives.csv | Reports the values of the ITC- and PTC-style credits applied for each electricity generation technology. The ITC-style credit isapplied to all potential generation and the PTC-style credit is applied only to used electricity. |
| params.csv | Reports several key input parameters plus the value of the objective function (an output): - year_ratio: The size of the model window in years, used to scale annuitized CAPEX + fixed OPEX costs. - max_imports: The maximum allowed imports as a fraction of total demand. - h: The time resolution of the model run in hours. - obj: The resulting value of the objective function. It is not recommended to use the value of the objective function as representative of costs; rather, the objective value can be useful when debugging or comparing scenarios, especially different model runs which should yield the same or similar results. |

Files detailing system design:

| File name | Description |
| --- | --- |
| Pipeline_Capacity.csv | Shows constructed pipeline capacities for each link. |
| Pipeline_Exists.csv: | Shows (as binary variables) whether a pipeline was constructed for each link. This file is only generated for mixed-integer optimizations or two-step solves which involve a full pipeline optimization. It is not generated if only a fully linear optimization is performed. |
| Generation_Capacity.csv | Shows the amount of capacity constructed at each node for each electricity generation technology. |
| Node_Capacities.csv | Shows the capacities of electricity generation technologies, hydrogen production, and hydrogen storage at each node (in their respective capacity units). It also shows a category, ‘H2_Unserved_Capacity’, which is the maximum amount of unserved hydrogen in a time period at each node. |

Files detailing system operation:

These files all contain details of how the system operates in each time period throughout the modeling window. Each has row indices of time periods and/or specific technologies and column indices of nodes or links. The data are the values of the specified parameter for that time period and/or technology at that node or link. Note for all link flow values, positive values indicate flow in the direction specified in the link name and negative values indicate flow in the opposite direction.

| File name | Description |
| --- | --- |
| Electricity_Used_raw.csv | The actual electricity generation used for hydrogen production for specific technologies. |
| Electricity_Used _Tech_Total.csv | The total amount of actual electricity generation used for hydrogen production in the entire model window for specific technologies. |
| Electricity_Used.csv | The total amount of actual electricity generation used for hydrogen production for all technologies. |
| Electricity_Curtailed_raw.csv | The amount of electricity generation that is curtailed for specific technologies. |
| Electricity_Curtailed_Tech_total.csv | The total amount of electricity generation curtailed in the entire model window for specific technologies. |
| Electricity_Curtailed.csv | The total amount of electricity generation that is curtailed for all technologies. |
| H2_Demand_Met.csv | The amount of hydrogen demand that is met in the model. |
| H2_Imports.csv | The amount of hydrogen imports. |
| H2_overserved.csv | The amount of overserved hydrogen. |
| H2_Unserved.csv | The amount of unserved hydrogen. |
| H2_Production.csv | The amount of hydrogen production. |
| Link_Flow.csv | The amount of total flow for each link. |
| Pipeline_Flow.csv | The amount of flow specifically via pipelines for each link. |
| Electricity_Potential_raw.csv | The potential for electricity generation for specific technologies. |
| Electricity_Potential_Tech_Total.csv | The total amount of electricity generation potential in the entire model window for specific technologies. |
| Electricity_Potential.csv | The total amount of electricity generation potential for all technologies. |
| Storage _Charge.csv | The amount of hydrogen that is injected into or withdrawn from storage for each storage technology. Positive values represent injection into storage and negative values represent withdrawals from storage. |
| Storage_Level.csv | The current amount of stored hydrogen for each storage technology. |
| Truck_Cost.csv | The levelized cost of hydrogen transport via tank-based transmission. |
| Truck_Flow.csv | The amount of flow specifically via tank-based transmission for each link. |

There are some model variables, such as pipeline costs or storage costs, that are not explicitly reported. If desired, these could be easily added by a user following the same format as other output writing.

Suggested use of outputs

HYPSTAT reports a very large number of outputs detailing system design and operation that can pose challenges to sort through. We propose some high-level suggestions for how to use the model outputs to glean valuable insights. See the example Case Study documentation for more detailed information on possible uses for outputs.

- Perform some high-level checks to make sure the model performed properly, e.g.:

  - Examine hydrogen overserved and hydrogen unserved to make sure that these are all zero, or you understand why they are not zero. Double check using the Demand_Met.csv outputs that your system is meeting demand properly.

  - Perform a high-level mass balance to make sure that the total amount of production and imports equals the amount of demand met and exports, factoring in any overserved or unserved hydrogen if they are non-zero.

- Use the reported capacities of each technology at nodes and links to understand the design of the system. Some useful guiding questions could be:

  - Which technologies are primarily used to produce hydrogen?

  - Does the model output tend to locate production and storage at locations of high demand, or does it locate them elsewhere and use hydrogen transmission?

  - Where (if at all) are hydrogen pipelines used?

- Consider the capacities and operating profiles for hydrogen production and electricity production. Is oversizing of electricity generation or hybridization of different generation technologies (e.g., wind and solar) occurring? Are there significant amounts of curtailment? If part of the model, when is grid electricity used?

- Consider the profiles for hydrogen storage charging and hydrogen storage levels. Is storage used significantly, and on what time scales are each storage technologies primarily used (e.g., hourly, daily, weekly, seasonal)?

- Consider hydrogen pipeline and tank-based transmission flows. At what times are each used to move hydrogen and where? Which direction does flow occur for each link? Are there any links or pipelines that have flow in both directions, depending on the time?

It can also be useful to use outputs from HYPSTAT to estimate system costs. We recommend using capacity outputs to calculate capital and fixed operating costs. Relevant operating outputs can be used to calculate variable operating costs. We recommend dividing these costs over amounts of hydrogen produced to estimate levelized costs for the whole system or at a nodal level. It can be insightful to look at how different technologies or value-chain steps (production, transmission, storage) contribute to levelized hydrogen costs.

A Simple Case Study

As an example, we provide a simple case study to demonstrate the modeling capabilities of HYPSTAT. We develop a hypothetical system (not based on any real location or data) with hydrogen demand and potential supply and transmission options to capture the primary features and abilities of HYPSTAT. We suggest using the Case Study for initial tests to ensure that HYPSTAT runs properly, and the input files for the Case Study can also be used as templates for developing custom scenarios.

The case study examines the infrastructure network for hydrogen demand, production, storage, and transmission across various nodes, each with distinct characteristics and demand profiles. The demand nodes evaluated include a connection point, industrial site, city, inland area, suburb, and coastal area. Different nodes have different potential for electricity generation—such as onshore wind, offshore wind, solar, nuclear, ang grid energy—export/import capacity, and storage capabilities.

This documentation also provides an overview of key concepts, formulas, and methodologies needed to derive essential results from HYPSTAT for hydrogen system balancing and dynamics. By analyzing direct model outputs, such as renewable, electrolyzer, transmission, and storage builds and locations, as well as by calculating Total CAPEX (Capital Expenditure), OPEX (Operational Expenditure), Levelized Hydrogen Cost (LCOH), Levelized Cost of Electricity (LCOE), and Capacity Factors for both electrolyzers and renewable resources, users can evaluate the technical and economic performance of hydrogen production facilities based on renewable energy sources and optimize the system for cost-effective, large-scale hydrogen production.

Case Design

Nodes and Network

Table X summarizes the nodes for the system and the demand, supply, storage, and import/export considerations for each node.

| Node | Descriptor | Demand | Onshore Wind | Solar | Offshore Wind | Nuclear | Grid | Export | Import | Geologic Storage | Tank Storage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Connection point | NO | YES (Good) | YES (Poor) | NO | NO | YES (Wholesale, lower) | YES | YES | NO | YES |
| B | Industrial site | YES | YES (Good) | YES (Poor) | NO | YES | YES (Fixed industrial) | NO | NO | NO | YES |
| C | City | YES | NO | NO | NO | NO | NO | NO | YES(?) | NO | NO |
| D | Inland | YES | YES (Poor) | YES (Good) | NO | NO | YES (Wholesale, higher) | NO | NO | YES (20 kT) | YES |
| E | Suburb, peaker plant | YES | NO | YES (Good) | NO | NO | YES (Fixed residential) | NO | NO | NO | YES |
| F | Coastal | YES | NO | YES (Good) | YES | NO | YES (Wholesale, higher) | NO | NO | NO | YES |

Detailed descriptions of each node are described below.

Node A (Connection Point): This node serves as a major junction in the hydrogen network, with no local demand but significant import and export capabilities. It has good onshore wind resources and modest solar potential. It connects to the wholesale grid at a lower price and has tank storage facilities.

Node B (Industrial Site): Characterized by a relatively large and flat demand profile, this site supports both onshore wind and solar (though with poor offshore wind potential). It pays a fixed industrial price, allowing it to manage stable hydrogen production and storage.

Node C (City): The city experiences medium demand, peaking in winter for heating and maintaining a consistent baseline for fleet vehicles. It lacks renewable energy resources, grid electricity, or hydrogen storage facilities to meet local H2 demand; all H2 is expected to be met by imports from other regions.

Node D (Inland): With small demand primarily for heavy-duty fuel cell electric vehicles (HD FCEVs), this node has poor onshore wind resources but good solar potential. It connects to the wholesale grid at a higher price and has significant geologic storage (20 kT) alongside tank storage.

Node E (Suburb, Peaker Plant): This node has a large demand, especially in summer for power generation. It has good offshore wind potential and a fixed residential grid connection, though it lacks geological storage.

Node F (Coastal): The coastal node features medium demand for port operations with relatively flat consumption patterns. It harnesses good offshore wind resources and is connected to the wholesale grid at a higher price. Tank storage is available, but there is no geologic storage.

The hypothetical system network and the relative locations of each node are shown in Figure X. This structured network serves as input for potential locations and connections. The HYPSTAT evaluation aims to identify optimal configurations and strategies for hydrogen production, storage, and distribution, ensuring the lowest system-wide costs (objective function).

Technologies and Cost References

HYPSTAT requires cost inputs for each technology considered. Table X shows the technologies considered in this Case Study and the references we use for cost inputs.

| Type | Tech | Cost Reference |
| --- | --- | --- |
| Generation | Onshore wind | NREL ATB moderate |
| Generation | Offshore Wind | NREL ATB moderate |
| Generation | Solar | NREL ATB moderate |
| Generation | Nuclear | LCOE from H2@Scale |
| Generation | Grid | NREL Cambium (Mid-case) for US national in hourly resolution (column name: ‘total_cost_enduse’) |
| Production | PEM Electrolyzer | H2A |
| Storage | Geologic | US number similar to NYSERDA, reference check with Jesse |
| Storage | Tank | US number similar to NYSERDA, reference check with Jesse |
| Transmission | Pipeline | US number similar to NYSERDA, reference check with Jesse |
| Transmission | Truck | US number similar to NYSERDA, reference check with Jesse |

Potential Considerations for Results Processing

Hydrogen Cost Calculations: Total CAPEX, OPEX, and Levelized Costs

Capital Expenditure

It can be useful to estimate the total capital expenses (CAPEX) to construct the optimal system identified by a HYPSTAT run. The total CAPEX represents the upfront cost required to build and install hydrogen production facilities, including the costs of renewable energy plants, electrolyzers, storage units, and pipelines.

Total CAPEX= (CAPEXGeneration+CAPEXElectrolyzer+CAPEXStorage+CAPEXTransmission )

For electricity generation systems, electrolyzers, and storage units, build costs are inputs that represent the initial cost per unit capacity (e.g., dollars per kW or kg) for each type of technology used. These costs can be derived from historical data, industry reports, or project-specific estimates. HYPSTAT reports the capacities of each technology in each zone, which can be used with the build cost inputs to estimate CAPEX.

It can also be useful to estimate an annualized cost for CAPEX to use for estimating levelized costs and comparing to operating expenses. This can be done using a Capital Recovery Factor (CRF). The CRF is based on the Weighted Average Cost of Capital (WACC) and the recovery time for each technology:

CRF=WACC×1+WACCRecovery Time1+WACCRecovery Time-1

Annuitized CAPEX=CRF× Build Cost × Zone Capacity

For pipelines, we recommend determining CAPEX using a cost coefficient and a power function based on the pipeline's capacity (which depends on the cost reference source). This reflects how the cost scales with capacity. The formula used is:

Pipeline CAPEX$/km=pipeline cost coefficient×CapacityTon/daypipeline cost power​

Here, the pipeline cost coefficient is a parameter reflecting the base cost, and the pipeline cost power accounts for scaling effects. The total pipeline cost is obtained by multiplying the cost per km by the total pipeline distance. To convert the total pipeline CAPEX into an annual cost, annuitize it using the Capital Recovery Factor (CRF).

Operational Expenditure

OPEX represents the ongoing costs associated with operating and maintaining the hydrogen production system. This includes fuel costs (e.g., nuclear), maintenance, and operational labor for each component, such as renewable energy sources, electrolyzers, storage units, and the transmission system. OPEX is categorized into fixed OPEX and variable OPEX.

Total OPEX= (OPEXGeneration+OPEXElectrolyzer+OPEXStorage+OPEXTransmission )

OPEX is typically expressed as dollars per kW or kg, or as a percentage of CAPEX (pipeline fixed OPEX), depending on the specific technology.

For trucks, HYPSTAT is set up to use a levelized cost which captures both CAPEX and OPEX on a per kg and distance basis.

For compression costs, they should be accounted for twice: once for the pipeline (to increase pressure from 20 bar, as output from the electrolyzer, to 70 bar for pipeline transport) and once for storage (to increase pressure from 50-70 bar to 400 bar for storage)

Levelized Hydrogen Cost (LCOH)

The Levelized Cost of Hydrogen (LCOH) represents the average cost per unit of hydrogen produced, accounting for all capital and operating expenses over one analysis year. It combines both CAPEX and OPEX, divided by the total amount of hydrogen produced. For systems with access to imported hydrogen, the imported amount also bears the cost of utilizing storage and transmission infrastructure.

LCOH= (CAPEX+OPEX)Hydrogen Amount​

The LCOH provides a single cost metric for comparing different scenarios. It can also be useful to examine the different contributions to LCOH from different components: for example, calculating an LCOH contribution with only the CAPEX and OPEX for hydrogen storage.

System Design

Locations and Capacities of Electricity Generation and Electrolyzers

Renewable Energy Capacity

The build capacity of renewable energy (RE) sources—such as solar and wind (onshore and offshore)—reflects how the model builds dedicated off-grid resources specifically for hydrogen production. The installed capacity is the maximum amount of electricity these sources can generate within a given operating hour. This is a model output based on direct inputs. Capacity determines the total potential for renewable energy production, but actual production is dependent on the capacity factor.

Electrolyzer Capacity

Electrolyzer capacity dictates how much hydrogen can be produced from electricity. This capacity directly affects the hydrogen production amount and rate, which is limited by both available electricity and electrolyzer capacity. This is also a direct model output.

Buildout Map

The buildout map visualizes the locations of renewable energy installations, electrolyzers, pipelines, trucks (transmissions), and storage units, along with their capacities. This helps optimize the system by coordinating and reducing system-wide costs between components, ensuring an efficient supply of electricity to the electrolyzers, optimal hydrogen production and storage, and efficient hydrogen supply to demand centers. The model outputs provide all results for maps directly, including the direction of the transmission infrastructure.

Storage Buildout and Capacity

Tank storage is typically used for balancing shorter-term daily or hourly hydrogen demand. In contrast, geological storage is designed to meet longer-term seasonal hydrogen demand, with charging occurring in the shoulder season and meeting demand during peak months. Storage technologies are crucial for managing supply and demand in the hydrogen production process. Storage capacity is generally measured in kilograms (kg) or kilotonnes (kT) for hydrogen storage. These systems ensure continuous hydrogen production even during periods of low renewable generation. The storage capacity build is a direct output of the model.

Transmission Buildout

The transmission infrastructure includes pipelines, trucks, and other delivery methods such as railways and marine transport (user-defined). It is required to transport hydrogen from generation sites to demand nodes or to storage units that meet hydrogen demand at various times and locations. The transmission system’s installed capacity, flow amounts, and direction at each temporal granularity are direct outputs of the model.

Operating Dynamics for RE, Electrolyzers, Transmission, and Storage

Electricity Generation Operation

Renewable energy sources operate intermittently depending on resource availability (e.g., sunlight for solar, wind speed for wind farms). Their operation is governed by installed capacity and capacity factor. Actual energy output depends on hourly or seasonal resource availability and weather patterns. This temporal profile is a direct output of the model.

Electrolyzer Operation

Electrolyzers operate based on the availability of electricity from renewable sources and grid electricity (if available). Their operational efficiency and capacity determine hydrogen production. The user sets the electrolyzer efficiency as an input. This temporal profile for electrolyzer is also a direct output of the model.

Storage Operation

Storage systems store excess hydrogen and discharge it when required. Their operation depends on the state of charge/discharge as well as technology-specific charge and discharge limitations.

Levelized Cost of Electricity (LCOE) and Capacity Factors

Levelized Cost of Electricity (LCOE)

The Levelized Cost of Electricity (LCOE) represents the cost per unit of electricity generated by renewable sources, considering CAPEX, OPEX, and total electricity production over the analysis year.

LCOERE=Total CAPEX+Total OPEX​Total RE electricity produced

This metric is useful for comparing the cost-effectiveness of different renewable energy technologies.

Capacity Factor (CF)

The Capacity Factor measures the actual output of a power generation technology compared to its maximum possible output.

CFRE=Total RE electricity produced Installed Capacity×8760

CFelectrolyzer=Total RE electricity consumed Installed Electrolyzer Capacity×8760

Capacity factors help in assessing the utilization of installed capacity and optimizing system performance.

Glossary

| Term | Definition |
| --- | --- |
| First term | Definition of term. Body text is Times New Roman 12. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. |
| Second term | Definition of term. Body text is Times New Roman 12. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. |
| Third term | Definition of term. Body text is Times New Roman 12. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. Enter body text. |

References

Bodini, Nicola, Dino Zardi, and Julie K. Lundquist. 2017. “Three-Dimensional Structure of Wind Turbine Wakes as Measured by Scanning Lidar.” Atmospheric Measurement Techniques 10 (8): 2881–2896. https://doi.org/10.5194/amt-10-2881-2017.

Zhu, Lei, Jacob Holden, Eric Wood, and Jeffrey Gender. 2017. “Green Routing Fuel Saving Opportunity Assessment: A Case Study Using Large-Scale Real-World Travel Data.” Presented at the 2017 IEEE Intelligent Vehicles Symposium (IV), Los Angeles, California, June 11–14. https://doi.org/10.1109/IVS.2017.7995882.

Appendix A: HYPSTAT Optimization Formulation

This appendix describes the detailed formulation of the optimization problem that HYPSTAT designs and solves to determine optimal hydrogen system design and operation.

Index Sets

| Name | Name in code | Description | Potential Values |
| --- | --- | --- | --- |
| t | m.T | Time periods of the year for which system dynamics are modeled. | Each hour, 2-hr period, day, etc. of the year. |
| N | m.Zones | The set of nodes for the system to model. | Arbitrary objects. We recommend string names, e.g., alphabetic qualifiers: ‘A’, ‘B’, ‘C’, etc. |
| L | m.Links | The set of potential transmission links between nodes. Note that they are inherently bi-directional. | String names that note which nodes the link goes between with the phrase ‘ to ‘. E.g., ‘A to B’. Note that they are inherently bi-directional, so if ‘A to B’ is a node, ‘B to A’ does not need to be in the set. |
| G | m.Gen_Techs | Generation technologies which the model can use to generate (or purchase) electricity to produce hydrogen. | Arbitrary objects. We recommend string names without spaces, e.g., ‘solar_pv’, ‘offshore_wind’, etc. |
| S | m.Stor_Techs | Hydrogen storage technologies which the model can use. | Arbitrary objects. We recommend string names without spaces, e.g., ‘tank’, ‘salt_cavern’, etc. |
| R | m.Gen_Tranches | The tranche options for generation technologies, i.e., representing supply curves for renewable build out where capacity factor drops as build-out increases. Should cover up to the technology with the most tranches; not every technology needs to go out to the furthest tranches. | Numeric tranches, e.g., 0, 1, 2, etc. |

Variables

| Name | Name in code | Description | Indices | Bounds/Domain | Units |
| --- | --- | --- | --- | --- | --- |
| Capacity variables |  |  |  |  |  |
| ECap | m.Gen_Capacity | The capacity of each generation technology in each tranche built in each node. | G,N,R | x≥0 | kW |
| HCap | m.Electrolyser_Capacity | The electrolyzer capacity built in each node. | N | x≥0 | kW |
| SCap | m.Storage_Capacity | The storage capacity of each storage technology built in each node. | S,N | x≥0 | kg |
| PCap | m.Pipeline_Capacity | The pipeline capacity built for each link. | L | x≥0 | kg/period |
| PEx | m.Pipeline_Exists | Whether or not a pipeline is built at all at each link. Only used in runs which pipelines locations are explicitly optimized. | L | 0, 1 | N/A |
| Electricity generation variables |  |  |  |  |  |
| EGen | m.Electricity_Potential | The potential amount of electricity that could be produced or purchased in each time period by each technology. | G,N, t | x≥0 | kWh |
| EUsed | m.Electricity_Used | The amount of electricity that is used in each time period by each technology for hydrogen production. | G,N, t | x≥0 | kWh |
| ECurt | m.Electricity_Curtailed | The amount of electricity that is curtailed (or not purchased) in each time period by each technology. | G,N, t | x≥0 | kWh |
| Hydrogen production variables |  |  |  |  |  |
| HProd | m.H2_Production | The amount of hydrogen produced at each node in each time period. | N, t | x≥0 | kg |
| HDem | m.H2_Demand_Met | The amount of hydrogen demand that is met (i.e., the amount of hydrogen consumption for end uses) at each node in each time period. | N, t | x≥0 | kg |
| HUn | m.H2_Unserved | The amount of hydrogen demand (e.g., for end uses) that is met/not provided in the model. | N, t | x≥0 | kg |
| HOv | m.H2_Overserved | The amount of excess hydrogen available at each node in each time period. | N, t | x≥0 | kg |
| HIm | m.H2_Imports | The amount of hydrogen imported from outside the system at each node in each time period. | N, t | x≥0 | kg |
| HEx | m.H2_Exports | The amount of hydrogen exported to outside the system at each node in each time period. | N, t | x≥0 | kg |
| Hydrogen storage variables |  |  |  |  |  |
| SCh | m.Storage_Charge | The amount of hydrogen charged into storage for each storage technology at each node in each time period. Positive values represent hydrogen injected into storage and negative values represent hydrogen withdrawn from storage. | S,N, t | R | kg |
| SLev | m.Storage_Level | The amount of hydrogen stored for each storage technology at each node at a given time period. | S,N, t | x≥0 | kg |
| SOpex | m.Storage_Charge_OPEX | The operating cost for charging storage for each storage technology at each node in each time period. | S,N, t | x≥0 | $ |
| Transmission variables |  |  |  |  |  |
| LFlow | m.Link_Flow | The hydrogen flow between zones for each link in each time period. Positive values represent flow in the direction specified by the link name and negative values represent flow in the opposite direction (e.g., for link ‘A to B’, positive values represent flow to node B and negative values represent flow to node A). | L, t | R | kg |
| PFlow | m.Pipeline_Flow | The hydrogen flow between zones for each link in each time period specifically via pipelines. Positive and negative values have the same meaning described for generic link flow. | L, t | R | kg |
| TFlow | m.Truck_Flow | The hydrogen flow between zones for each link in each time period specifically via trucks. Positive and negative values have the same meaning described for generic link flow. | L, t | R | kg |
| POpex | m.Pipeline_Opex | The operating costs for hydrogen flow in pipelines for each link in each time period. | L, t | x≥0 | kg |
| TOpex | m.Truck_Cost | The levelized costs for hydrogen flow via trucks for each link in each time period. | L, t | x≥0 | kg |

Parameters

Constraints

Demand Constraints

The hydrogen demand input must be met by hydrogen in the model via the m.Demand_Met variable. This is enforced in a strict way, i.e., in every period, via strict_demand_rule(m, t, zone):

HDemN, t=Hydrogen demand inputt, N ∀ t, N

p

tt+pHDemN, t=tt+pHydrogen demand inputt, N ∀ N, t s.t. t%p=0

Import and Export Constraints

Total system imports must be within the specified import limit, and total system exports must be at least the amount specified. Also, imports and exports must be zero in zones which are not import or export zones.

| total_imports_rule(m) | N,tHImN,t≤maximports |
| --- | --- |
| zone_imports_rule(m, t, zone) | HImN,t=0 ∀ t, N for N not in import zones |
| total_exports_rule(m) | N,tHExN,t≥minexports |
| zone_exports_rule(m, t, zone) | HExN,t=0 ∀ t, N for N not in export zones |

Storage Constraints

For each storage technology, the storage level is related to the storage charging and discharging—i.e., the storage level in each period is equal to the previous level plus or minus whatever amount of hydrogen was withdrawn or injected. There are also constraints on the speed of storage charging and discharging related to the total storage capacities.

| Storage_multi_period_rule(m, stor_tech, t, zone) | SLevS,N,t=SLevS,N,t-1+SChS,N,t ∀ S, N, t≥2 SLevS,N,t=1=SLevS,N,t=last+SChS,N,t=1 ∀ S, N |
| --- | --- |
| Storage_charge_rule(m, stor_tech, t, zone) | SChS,N,t ≤Storage charge limitS×SCapS,N ∀ S, N, t |
| Storage_discharge_rule(m, stor_tech, t, zone) | SChS,N,t ≥ -Storage discharge limitS×SCapS,N ∀ S, N, t |

The storage level must not exceed the built storage capacity at any zone for any technology. Storage capacities at each node and in the total system must be less than the limits specified in the inputs.

| Storage_level_rule(m, stor_tech, t, zone) | SLevS,N,t≤SCapS,N ∀ S, N,t |
| --- | --- |
| Storage_capacity_cap_rule(m, stor_tech, t, zone) | SCapS,N ≤Nodal storage capacity limitS,N ∀ S, N |
| Total_storage_capacity_rule(m, stor_tech, t, zone) | NSCapS,N ≤Total storage capacity limitS ∀ S, N |

Note that the inputs for storage charge and discharge limits and storage capacity limits can be specified as infinite or left unspecified. For simplicity in model formulation, HYPSTAT checks for infinite or unspecified values and simply skips constraints which would involve these infinite or unspecified values. (Mathematically, this would be equivalent to formulating the constraint with the infinite value.)

Link Flow Constraints

Hydrogen flow for different links occurs via pipelines or truck flow. For simplicity in mass balances, we define a total link flow variable which is determined by the pipeline and truck flow via total_link_flow_rule(m, t, link):

LFlowL,t=PFlowL,t+TFlowL,t ∀ L,t

Pipeline and truck flows can occur in either direction and have specific capacity limits (HYPSTAT does not currently support different limits for forward and reverse flow). Pipeline flow is limited by the capacity of the pipeline built for a given link (another model variable), while truck flow is limited by the specified truck maximum capacity given in the inputs, along with a boolean for whether truck flow is allowed at all on that link. For simple constraint functions, the forward and reverse flow constraints are separated in HYPSTAT.

| forward_max_truck_rule(m, t, link) | TFlowL,t≤TruckmaxcapacityL×Truck flow allowed[L] ∀ L,t |
| --- | --- |
| reverse_max_truck_rule(m, t, link) | TFlowL,t≥-TruckmaxcapacityL×Truck flow allowed[L] ∀ L,t |
| forward_max_pipeline_rule(m, t, link) | PFlowL,t≤ PCapL∀ L,t |
| reverse_max_pipeline_rule(m, t, link) | PFlowL,t≥-PCapL∀ L,t |

Pipeline capacity in each link is limited by capacity inputs (similar to truck inputs) and, for mixed-integer optimizations, whether or not a pipeline is built at all for a given link.

| pipeline_exists_rule(m, link) | PCapL≤PipelinemaxcapacityL×Pipeline flow allowed[L]×PExL ∀ L |
| --- | --- |
| pipeline_size_rule(m, link) | PCapL≤PipelinemaxcapacityL×Pipeline flow allowed[L] ∀ L,t |

pipeline_exists_rule(m, link) is used for a mixed-integer optimization. For a fully linear optimization which only optimizes pipeline sizes and not locations, pipeline_size_rule(m, link) is used, typically after a coarse robust optimization to determine locations of pipelines, which are fed into the model via the Pipeline flow allowed[L] input.

Production Constraints

The potential for total electricity production in each time period is set by the production profiles (e.g., for a particular renewable resource) for each technology at each node and the capacity of that technology at each node. Of this electricity potential, the model can choose how much electricity to curtail and how much to use. To avoid negative electricity use, the curtailed electricity must be constrained to be less than or equal to the electricity production potential, and the electricity used is set by the production potential and the curtailment.

| Electricity_Potential_rule(m, tech, t, zone) | EGenG,N,t=RProduction profile[G,N,R,t]×ECapG,N,R ∀ G,N,t |
| --- | --- |
| Curtailed_electricity_limit_rule(m, tech, t, zone) | ECurtG,N,t≤ EGenG,N,t ∀ G,N,t |
| Electricity_Used_rule(m, tech, t, zone) | EUsedG,N,t= EGenG,N,t- ECurtG,N,t ∀ G,N,t |

In Electricity_Potential_rule(m, tech, t, zone), the Production profile[G,N,R,t] term can be though of as the normalized output or capacity factor in that time period for the electricity producer, e.g., set by wind, sun, or other weather patterns for wind or solar generation.

The maximum capacity of electricity generators for each technology, tranche, and node is restricted in the inputs (i.e., to represent the potential capacity of the physical system) and must be restricted via Gen_capacity_limit_rule(m, tech, zone, tranche):

ECapG,N,R≤Maximum capacityG,N,R ∀ G,N,R

Note that some technologies may not have capacity in all tranches, depending on the zone. In the case where a technology does not have a given tranche (e.g., a node has only one supply curve and thus only has tranche ‘0’), the above constraint is skipped for that tranche, technology, and node. This works because Electricity_Potential_rule(m, tech, t, zone) sets the potential electricity generation and gives no potential generation for tranches without capacity in a given node, thus the model has no reason to build that capacity.

Hydrogen production is set by the amount of electricity used and a conversion efficiency for electrolysis. The model must also ensure that for a given node, there is enough electrolyzer capacity to produce hydrogen; i.e., the amount of electricity used must be limited by the electrolyzer capacity:

| H2_Production_rule(m, t, zone) | HProdN,t=GEUsedG,N,tConversion efficiency[N] ∀ N,t |
| --- | --- |
| Electrolyser_Capacity_Limit_rule(m, t, zone) | GEUsedG,N,t≤HCapN×h ∀ N,t |

Power Balance Constraint

Perhaps the crux of the model formulation is the power/energy balance for hydrogen at each hour and in each location, set by H2_Balance_rule(m, t, zone). It follows the standard mass balance that all hydrogen produced, discharged from storage, and transmitted (generation + in) into a given node at a given time must be equal to the hydrogen consumed, charged into storage, and transmitted out (consumption + out) of that node at that time. We add two additional terms, “unserved” and “overserved” hydrogen, to this balance, essentially to cover the “accumulation” term of a typical mass balance.

HProdN,t+HUnN,t+LLFlowL,t×Link flow directionL,N+HImN,t= SSChS,N,t +HDemN, t+HOvN,t+HExN,t∀ N,t

The “Link flow direction” term is generated based on the inputs for each link and zone and represents whether flow is into or out of a zone. For example, for a link “A to B”, positive flow represents flow out of A and into B, and negative flow represents flow out of B and into A. In this case “Link flow direction[A to B, B]” would be 1, “Link flow direction[A to B, A]” would be 1, and “Link flow direction[A to B, C]” would be 0 since flow between A and B is irrelevant to the mass balance on C. In practice in the model, the summation over links is only applied to relevant links connected to the node in question, instead of summing over all links and using a link flow direction of 0 for irrelevant links.

Unserved hydrogen represents hydrogen demand that is not met by the model’s system design, and thus is a sort of additional “false production” of hydrogen; overserved hydrogen represents excess hydrogen that is produced but is not needed to meet any demand, and thus is a sort of additional “false consumption” of hydrogen. Both unserved and overserved hydrogen are given cost penalties, so for typical models we would expect both to be zero at all times and nodes. If this is not the case, this may indicate an error or infeasible system unless you are specifically using those terms in a different way.

Transmission and Storage Cost Constraints

Transmission and storage costs are unique in the model in that operating costs must be applied to either the absolute value of relevant variables (pipelines should incur positive operating expenses for flow in either direction) or have different costs applied to positive and negative values (different operating costs for charging vs. discharging storage). To maintain a linear model, these costs must be calculated as paired constraints.

For hydrogen transport, the operating cost is set as a cost for each kg of hydrogen flow in either direction—i.e., a multiplier on the absolute value of flow for each link. The cost is treated as a variable in the model but must be greater than both the positive and negative values of the operating cost times the flow for each link at each time period. The model tries to minimize this cost, but the result is that the minimum possible cost is the proper operating cost for the absolute value of the flow. This is implemented through sets of paired constraints for pipeline flow and truck flow.

| Forward_Pipeline_Opex_rule(m, t, link) | POpexL,t≥PFlowL,t×Pipeline Opex[link] ∀ L,t |
| --- | --- |
| Reverse_Pipeline_Opex_rule(m, t, link) | POpexL,t≥-PFlowL,t×Pipeline Opex[link] ∀ L,t |
| Forward_Truck_Cost_rule(m, t, link) | TOpexL,t≥TFlowL,t×Truck LCOT[link] ∀ L,t |
| Reverse_Truck_Cost_rule(m, t, link) | TOpexL,t≥-TFlowL,t×Truck LCOT[link] ∀ L,t |

Hydrogen storage behaves similarly, except that we only consider one operating expense on charging. Because of the conservation of mass in storage, any operating expenses on discharging can also be incorporated into this expense: every kg of hydrogen charged must be eventually discharged, and the time within the year that this occurs is irrelevant to the model as it only considers total expenses for the year. This is accomplished with a single constraint, Storage_Charge_Opex_rule(m, stor_tech, t, zone):

SOpexS,N,t≥SChS,N,t×Storage Charge OpexS,N∀ S,N,t

In this case, with the storage charge operating expense as a positive or zero variable, the model will try to set the cost to zero or as low as possible. On discharging, when SChS,N,t is negative, the model succeeds. On charging, the lowest the operating expense can be is the proper operating cost to represent that chargine.

Objective Function

The objective function for the HYPSTAT model is intended to represent all costs for the model year, including annuitized capital costs and operating expenses. As such, the model determines the system design which minimizes all of these costs, subject to constraints which ensure that hydrogen demand is met. We break this up into six types of costs, so that the model objective is:

Minimize Gen_Stor_CAPEX +Pipeline_CAPEX+Transmission_OPEX+Gen_Stor_OPEX+Incentives+Penalties

Each of the six types of costs are defined as:

            - Gen_Stor_CAPEX: The annuitized capital expenses and fixed operating expenses for electricity generation, hydrogen production, and hydrogen storage technology.

Gen_Stor_CAPEX =NR,GECapG,N,R×Generation CAPEX[G,N]+HCapN×Electrolyzer CAPEX[N]+SSCapS,N×Storage CAPEX[S,N]

            - Pipeline_CAPEX: The annuitized capital expenses and fixed operating expenses for hydrogen pipelines. This calculation is more complex than the others due to the varying CAPEX with size and a difference in calculation depending on whether pipeline locations are optimized.

First, a “raw CAPEX” value is calculated which represents the actual one-time cost of the pipeline:

Pipeline_raw_CAPEX=LPCapL×Capacity CAPEX+Fixed CAPEX [×PExL]×Link distance[L]

The bracketed portion where the fixed CAPEX is multiplied by the PExL variable is omitted if the model is not optimizing for pipeline locations. In that case, this acts as a fixed, unavoidable adder for all pipeline links, and thus does not affect the model optimization (though it does make the model objective value not accurate to actual system costs), but avoids the use of binary variables to speed up the optimization.

The “raw CAPEX” is then annuitized and fixed OPEX is added as a fraction of this raw CAPEX.

Pipeline_CAPEX=Pipeline_raw_CAPEX×CRF +Pipeline_raw_CAPEX×Fixed OPEX fraction

            - Transmission_OPEX: The operating expenses for pipeline transmission and truck transmission.

Transmission_OPEX =LtPOpexL,t+tTOpexL,t

            - Gen_Stor_OPEX: The operating expenses for electricity generation, hydrogen production, and hydrogen storage technology.

Gen_Stor_OPEX =NS,tSOpexS,N,t+G,t(EUsedG,N,t×Generation OPEXG,N,t)

            - Incentives: Incentives such as tax credits to be applied. Originally intended to represent different credits from the IRA, but could be used to represent other incentives as well.

Incentives=-G,N,tEUsedG,N,t×PTCG-G,N,tEGenG,N,t×ITCG

            - Penalties: Costs incurred from unserved or overserved hydrogen.

Penalties=NtHUnN,t×Unserved cost+tHOvN,t×Overserved cost